Affymetrix Eukaryotic Sample and Array Processing Protocol

# Protocol #: PA001_1

# Version 1

# Date: March 12, 2007

# GEC

# California Institute of Technology

Affymetrix Tech Support:  1-888-362-2447  Customer #126888

Fluidic Station Tech Support:  1-888-362-2447  Customer # FS1025

Bioanalyzer Tech Support:  1-800-227-9770

Quantification of RNA

Quantify total RNA sample on Nanodrop

Run ~50 ng of total RNA sample on Bioanalyzer

One Cycle cDNA Synthesis (reagents in blue boxes in -20)

Serial dilutions of Poly-A RNA Control stock in Poly-A Control Dilution Buffer

Poly-A Control Stock       	First	    Second	Third

2 ul				2:40	      1:50	1:50

The first dilution can be stored up to six weeks at -20°C

RNA/T7-Oligo(dT) Primer Mix

Sample RNA (2ug)			< 8 ul

Poly-A RNA controls, 3rd dilution	2 ul

50uM T7-Oligo(dT) Primer		2 ul

RNase-free Water			to 12 ul

Flick tubes to mix and centrifuge briefly

70°C, 10 min in thermal cycler

4°C, at least 2 min

First-Strand Master Mix

5X 1st Strand Reaction Mix	4 ul

0.1M DTT			2 ul

10mM dNTP			1 ul

Add 7ul to each RNA/T7 Primer mix for a final volume of 19 ul

42°C, 2 min in thermal cycler

Add 1 ul of Superscript II

42°C, one hour

4°C, at least 2 min

Second-Strand Master Mix- use thermal cycler.

RNase-free Water		91 ul

5X 2nd Strand Reaction Mix	30 ul

10mM dNTP			3 ul

DNA Ligase			1 ul

DNA Polymerase I		4 ul

RnaseH			1 ul

Add 130 ul Second-Strand master mix to each first-strand synthesis sample

16°C, 2 hours in thermal cycler  * set a timer

Add 2 ul of T4 DNA polymerase

16°C, 5 min

Add 10 ul of 0.5M EDTA.

Proceed to cleanup of double-stranded cDNA; do not leave at 4°C for long periods of time.

Cleanup of Double-stranded cDNA (reagents on center shelves)

Transfer double-stranded cDNA to 1.7ml tube

Add 600 ul cDNA Binding Buffer

Vortex; mixture should stay yellow (if it turns orange or violet, add 10 ul of 3M NaOAc, pH5.0)

Add 500 ul of sample to the spin column

Centrifuge for one min at 8000 x g; discard flow-through

Add remaining mixture and centrifuge; discard flow-through

Transfer spin column into a new 2 ml collection tube

Add 750 ul cDNA Wash Buffer (make sure ethanol has been added to buffer)

Centrifuge for one min at 8000 x g; discard flow-through

Centrifuge at max speed for 5 min to allow complete drying of the membrane

Transfer spin column into a 1.7 ml tube

Pipet 14 ul of cDNA Elution Buffer directly onto the membrane

Incubate for one min at room temp; Centrifuge for one min at max speed to elute

Biotin Labeling of Antisense cRNA (reagents in blue boxes in -20)

IVT Reaction Master Mix

RNase-free Water		8 ul

10X IVT Labeling Buffer	4 ul

IVT Labeling NTP Mix	12 ul

IVT Labeling Enzyme Mix	4 ul

Add 28 ul IVT Reaction Master Mix to cleaned double-stranded cDNA

37°C, 16 hours in thermal cycler

Cleanup of Biotin-Labeled cRNA (reagents on center shelves)

Transfer IVT reaction into a 1.7 ml tube

Add 60 ul RNase-free water

Vortex for 3 sec

Add 350 ul IVT cRNA Binding Buffer

Vortex for 3 sec.

Add 250 ul ethanol to the lysate

Mix well by pipetting

Apply sample to spin column

Centrifuge at 8000 x g for 15 sec; discard flow-through

Add 500 ul of IVT cRNA Wash Buffer

Centrifuge at 8000 x g	 for 15 sec; discard flow-through

Add 500 ul of 80% ethanol

Centrifuge at 8000 x g for 15 sec; discard flow-through

Centrifuge at max speed for 5 min to allow complete drying of the membrane

Transfer spin column into a 1.7 ml non-stick tube

Pipet 11 ul of RNase-free water directly onto the membrane

Centrifuge for one min at max speed to elute

Pipet another 10 ul of RNase-free water directly onto the membrane

Centrifuge for one min at max speed to elute

Quantification of the cRNA

Dilute 1 ul of sample 1:5

Use NanoDrop to quantify the cRNA

Run ~500 ng of sample on Bioanalyzer

If the graph looks good in the bioanalyzer, proceed to	 cRNA fragmentation

Can store cRNA at -20°C or -80°C

Fragmenting the cRNA (reagents in box with cleanup supplies on center shelves)

If sample has been frozen, use the Nanodrop to re-quantify prior to fragmentation

If the sample looks good, proceed to fragmentation

Fragmentation Reaction

cRNA(20 ug)			< 32 ul

5X Fragmentation Buffer	8 ul

RNase-free water		to 40 ul

94°C, 35 min in thermal cycler

Put on ice following incubation

Load 1ul of fragmentation reaction directly into the bioanalyzer

Hybridization for Probe Array  (reagents in blue box in -20)

Preheat heat blocks to 65°C and 99°C

Turn on oven, set to 45°C

Equilibrate probe array to room temperature before use

Heat the 20X Euk. Hyb. Control at 65°C for 5 min in heat block

Hybridization Cocktail

Standard Array     	Midi Array     		Mini/Micro Array

Fragmented cRNA	(15 ug) 30 ul            	(10 ug) 20 ul       	(5 ug) 10 ul

Control Oligo B2	5 ul			3.3 ul			1.7 ul

20X Euk.Hyb.Control	15 ul			10 ul			5 ul

Herring Sperm DNA	3 ul			2 ul			1 ul

BSA(50mg/ml)	3 ul			2 ul			1 ul

2X Hyb Buffer	 (fridge)150 ul			100 ul			50 ul

DMSO	 (hood)		30 ul			20 ul			10 ul

ddH2O			64 ul			42.5 ul	  		21.3 ul

300 ul			200 ul			100ul

99°C, 5 min in heat block

Wet the (room temperature) array by filling it through the lower septa with appropriate volume of 1X Hybridization Buffer.  Insert another pipet tip in the upper septa for venting.

Standard Array	250 ul

Midi Array		180 ul

Mini/Micro Array	100 ul

Put probe array at 45°C for 10 min in oven

Move hyb cocktail to 45°C oven for 5 min

Spin hyb cocktail at max speed for 5 min to remove any insoluble material from the hyb mixture

Remove the buffer solution from the probe array cartridge and fill it with the hyb cocktail

Put back in the 45°C oven and set it to rotate at 45 rpm

Hybridize for 16 hours

Washing, Staining, and Scanning

(If the array pattern is not in the database, need to create it before adding a project. Go to affymetrix.com.  Log in.  Click on Support.  Click on GeneChip Arrays.  Look for the desired array pattern.  Download the Library Files, save in the desktop.  To add the probe array type in GCOS:  Open the library files in the desktop.  Open Full file.  Look for the Setup.exe Application Type.  Then follow the installation process.  To upload the pattern in Resolver:  Open the library files, copy and paste into a new folder the .cdf and .gin files.  Open Resolver. Go to Admin.  Create the new species in the New Species Database if it’s not under Species.  Choose menu File -> Import ->Pattern.  Select Affymetrix/GeneChip (CDF/CDL Files).  File Name: browse for location of .cdf file.  Pattern Name: type in the array name.  Species Database: scroll down for the species just created in the database.  Look for the feature size.  Click on Import.)

Turn on the Fluidics Station

Change the intake deionized water reservoirs to their respective solutions Wash Buffers A & B (make sure there's at least 200ml of Wash Buffer A)

In the computer, click on the GeneChip Operating Software icon.  Click on the Experiment icon.  Create a project name (initials_ date).  Choose the array genome used. Create an experiment name.  Sample name and Sample type fields are the same as the experiment name.  Click on Save.  Do the same procedure for other samples.  Click on the Fluidics icon.  Click on Module 1.  Choose Prime for the protocol. Then click on Run.  Do the same procedure for the other samples.  Priming the Fluidics Station will take about 10 min.

Prepare staining solution (reagents in white box in fridge)

Stains 1 & 3: SAPE Solution Mix (per sample)

2X Stain Buffer	600 ul

BSA 50 mg/ml	48 ul

SAPE 1 mg/ml	12 ul

ddH2O			540 ul

1200 ul

Vortex.  Divide into 2 aliquots (600ul each)

Stain 2: Antibody Solution Mix (per sample)

2X Stain Buffer		300 ul

BSA 50 mg/ml		24 ul

Goat IgG Stock 10 mg/ml	6 ul

Biotinylated Ab 0.5 mg/ml	3.6 ul

ddH2O				266.4 ul

600 ul

Take probe array out of 45°C oven

Remove hyb cocktail from chip, and store in -20 freezer

Fill chip with Wash Buffer A

Insert the appropriate probe array into the designated module from the fluidics station (make sure the cartridge lever is in the eject position)

When finished, move the cartridge lever back to the engage position

In the computer, click on Module 1.  Look for the experiment name that corresponds to the probe array in module 1.  Select the appropriate protocol to run (will find this in the enclosed insert with the probe arrays).  Then choose Run to begin the washing and staining.

| Chip | Fluidics Protocol for FS400 |
| --- | --- |
| Human Genome U133 Plus 2.0 | EukGE-WS2v5 |
| Mouse Genome 430 2.0 | EukGE-WS2v5 |
| Rat Genome 230 2.0 | Midi_euk2v3 |
| Human Genome U133A 2.0 | Midi_euk2v3 |
| Mouse Genome 430A 2.0 | Midi_euk2v3 |
| Canine 2.0 | Midi_euk2v3 |
| Drosophila Genome 2.0 | Midi_euk2v3 |
| C. elegans Genome Array | EukGE-WS2v4 |

Follow the instructions on the LCD window on the fluidics station.  When asked, change empty vial to stain solution 1 (SAPE soln).  After about 45 min, change stain solution 1 to stain solution 2 (Ab soln).  Then after 5-15 min, change Ab soln to stain solution 3 (SAPE soln).

Turn on scanner. Let it warm up for 10-15 min before using.

Remove probe array from the fluidics station module.  Check the probe array window for air bubbles.   If large bubbles are present, fill the array with Wash Buffer A manually using a micropipette.

Follow the instructions on the LCD window on the fluidics station.  Remove stain solution vial from the fluidics station and replace it with an empty vial.  The fluidics station automatically performs a Cleanout procedure.  In the computer, choose Close; do not choose Stop.  When the cleanout is finished, change intake wash buffer A & B reservoirs back to the ddH2O containers.  Turn off station when finished.

In the computer, click on the Start scan button.  Select the experiment name of the sample.   Choose Load/Eject to insert the array in the scanner.  Click on Start.  Click Ok.   The file name is “.DAT”.

After scanning, click on the Start button, choose Load/Eject to remove the probe array.  Turn off the scanner.

Check the probe array image for B2 Oligo:  alternating pattern on the border,  checkerboard pattern at each corner, and the array name.

Data Acquisition

1. Create Report:

Run GeneChip Operating System program

On left side of screen, look for experiment name

Look for .CEL file nested under experiment name (click on + to left of name)

Right click .CEL

Choose Analyze

Click OK at suggested file name

Right click .CHP (nested under .CEL)

Choose Report

Print Report

Properties -> Advanced -> Finishing: 2 page per sheet

Compare values to make sure they pass the QC.

Check that they’re consistent between 	samples.

Background: average values should be between 40-60.

Number Present: should be between 25%-65%.

Number Marginal:  should be below 2%.

Housekeeping Controls: for both Actin and GAPDH, the Sig(3’/5’) should be < 3 for one-cycle assay; higher ratios for two-cycle assay.

Spike Controls:  for BioB, BioC, BioD, and Cre, should be present “P”

2.  Create CAB File:

Run Data Transfer Tool program

Choose Transfer Out -> CAB Files

Choose Transfer Data Out

Choose Transfer GCOS Data

Change Project name to the correct project

Click on project name

Click Review

Click Start

CAB file will end up on desktop

There must be .EXP and .CEL files for each scan

Create new folder

Copy CAB contents into it

3.  Get Barcodes:

Go to gec.bio.caltech.edu

Choose Caltech Extensions

Choose Chip Barcode Generator

Enter Username

Set Resolver Group as name of lab

Choose number of barcodes desired

Select and copy generated barcodes

4.  Import Information into Resolver:

Log into Resolver

I.  Choose Annotation -> Chips -> New

Chip Type: Affymetrix GeneChip

Pattern Name: search for correct species

Access Control: Public, GEC, lab name

(now is a good time to Duplicate to desired number of chips)

Chip Barcode: paste copied barcodes

Click Save and Close

II.  Choose Annotation -> Preps -> New

Cells/Tissues: mRNA UNKNOWN

Prep Name = Prep Code: name of each sample, starting with your initials

Click Save and Close

III.  Choose Annotation -> Intensity Hybs -> New

Access Control: Public, GEC, lab name

Chip Barcode: search for newly-created ones (under current date)

Prep Code: search for newly-created ones (type in intials)

will automatically fill in Hyb Name

Click Save and Close

IV.  Choose menu File -> Import -> Scan -> Intensity

Select Affymetrix/GeneChip (CDF/CEL Files)

Scan Quality: Passed QC

Chip Barcode: search for newly-created ones

will automatically fill in Scan Name

File Name: browse for location of .CEL file (in the folder on the desktop created 		earlier)

Check to make sure File Name matches with Hyb!

Click Import

To check the scan queue, choose menu File -> Monitor

Affymetrix Reagent Preparation- Eukaryotic Sample

12X MES Stock Buffer (1 L)- 1.22M MES, 0.89M Na+

MES hydrate				64.61 g

MES Sodium Salt			193.3 g

Molecular Biology Grade Water	800 ml

*Mix and adjust volume to 1L

Check that the pH is between 6.5 to 6.7.

Filter through a 0.2 um filter in the cabinet by Vijaya's desk. Do not autoclave.

Store at 4°C and protect from light.  Discard solution if yellow.

2X Hybridization Buffer (50 ml)-  Final 1X is 100mM MES, 1M NA+, 20mM EDTA, 0.01% Tween-20

12X MES Stock Buffer (fridge)	8.3 ml

5M NaCl				17.7 ml

0.5M EDTA (far shelves)		4 ml

10% Tween-20 (far shelves)		0.1 ml (undiluted on middle shelves, brown bottle)

ddH2O					19.9 ml

*Store at 4°C and protect from light.

1X Hybridization Buffer

Dilute 2X Hybridization buffer with ddH2O in 1:2 dilution.

*Store at 4°C and protect from light.

Wash Buffer A: Non-Stringent Wash Buffer (1 L)- 6xSSPE, 0.01% Tween-20

20X SSPE (yellow label)	300 ml

10% Tween-20 (far shelves)	1 ml (undiluted on middle shelves, brown bottle)

ddH2O				699 ml

*Filter through a 0.2um filter in the cabinet by Vijaya's desk.  Store at 4°C.

Wash Buffer B: Stringent Wash Buffer (1 L)- 100mM MES, 0.1M Na+, 0.01% Tween20

12X MES Stock Buffer	83.3 ml

5M NaCl			5.2 ml

10% Tween-20		1 ml

ddH2O				910.5 ml

*Filter through a 0.2 um filter.

Store at 4°C and protect from light.

2X Stain Buffer (250 ml)- Final 1X is 100mM MES, 1M Na+, 0.05% Tween-20

12X MES Stock Buffer	41.7 ml

5M NaCl			92.5 ml

10% Tween-20		2.5 ml

ddH2O				113.3 ml

*Filter through a 0.2um filter  in the cabinet by Vijaya's desk.

Store at 4°C and protect from light.

Goat IgG Stock (5 ml)-  10 mg/ml (undiluted stock is in orange capped vial in freezer)

Resuspend 10 mg in 1 ml of 150mM NaCl

*Store at 4°C

Biotinylated Anti-Streptavidin (1 ml)- 0.5 mg/ml

Resuspend 0.5 mg in 1 ml of ddH2O.  *Store at 4°C.

Regents Required:

Affymetrix:

- 900493 GeneChip® Expression 3' Amplification One-Cycle Target Labeling and

Control Reagents (includes 900433 GeneChip® Eukaryotic Poly-A RNA Control Kit; 900431 One-Cycle cDNA Synthesis Kit; 900449 IVT Labeling Kit; 900454 GeneChip® Expression 3' Amplification Reagents – Hybridization Controls; 900371 GeneChip® Sample Cleanup Module)

Invitrogen:

-  15561020 BOVINE SERUM ALBUMIN (BSA) 150 MG

-  S866 STREPTAVIDIN, R-PHYCOERYT 1 ML

SIGMA:

-  I5256-10MG GOAT IMMUNOGLOBULIN G (IGG) FROM SERUM 10mg

Fisher Scientific:

-  PRD1811 Sperm, Herring, DNA, 10mg (in plastic bag on top shelf of freezer)

Vector Laboratories:

-  BA-0500 Biotinylated Anti-Streptavidin 0.5 MG

Agilent Technologies:

-  5067-1511 Agilent RNA 6000 Nano Kit 2